Isocyanate & Polyol  travel path at Huyton

- ISO/Poly is delivered by road tankers and offloaded (see document Tanker offload work instruction)

- ISO is stored in bulk temperature controlled tanks. The tanks are;

- Temp controlled

- Low content audio visual warning system

- High content audio visual warning system

- Very high content shut off system

- Bunded to 110% of max volume

- Vapour return unit to return any vapor to the tanker

- ISO and POLY are pumped from the tankers to the “day tanks” to be mixed

- Pumped under pressure into jigs where they cure;

A) it takes 5 minutes for the foam to set the foam is then considered inert

Any  potential release of isocyanates is from the injection into the jig  is controlled via LEV system and regularly monitored (see doc EPN7406)

OSB travel path Huyton

- OSB is brough in generic sizes and stored internally in the warehouse

- OSB is cut down to specific sizes with a drop saw

- Anywhere where there is a risk of dust in the factory has  LEV extraction the dust is collected internally and sent for recycling

SOLVENT travel path Huyton

- Solvent is delivered in a 200ltr steel drum

- Stored externally in a fire proof bunded chemical cupboard

- Manually pumped into a fluorinated container to put into a solvent bath internally. Solvent baths are closed and covered with a LEV hood

- Waste solvent is stored in a bunded waste IBC externally and disposed of by professional waste management company

Process flow