C2 Emissions

C2 A

Process stages

| Process stage | Expected emissions | Control measures in place | Substance | Maintenance |
| --- | --- | --- | --- | --- |
| Delivery of materials | No emissions expected | Vapour return unit on bulk storage tanks | Di-isocyanate as total NCO group and VOC blowing agents | Annual inspections for air quality and extraction systems carried out |
| Daily start up | No emissions expected | Enclosed system cycles product until use | Di-isocyanate as total NCO group and VOC blowing agents |  |
| Injection of foam | Fugitive emissions expected | Air quality checks <.0002mg/m3 | Di-isocyanate as total NCO group and VOC blowing agents | Annual inspections for air quality and extraction systems carried out |
| Curing of product | Fugitive emissions expected | Dependent on thickness foam can take up to 10 minutes to cure this is done inside the press, air quality checks done @ < .0002mg/m3 | Di-isocyanate as total NCO group and VOC blowing agents | Annual inspections for air quality and extraction systems carried out |
| Quality inspections | Fugitive emissions expected | Any adjustment to panels are completed under filtered LEV’s to contain the dust Air qual results Between 0.19 and 0.72 mg/m3 | Particulate Matter | Annual inspections for air quality and extraction systems carried out |
| Cleaning | Fugitive emissions expected | Foam head is soaked in solvent under LEV extraction (see pic1) air quality tests completed @<.42mg/m3 | 2-Butoxyethanol | Annual inspections for air quality and extraction systems carried out |
| Waste | No emissions expected | Any waste that cannot be repurposed in house will be processed by an authorised waste disposal company under code 17-09-04 or sent for recycling |  |  |
| Close down | No emissions are expected | Enclosed system cycles product until use |  |  |
| General operation Odour | No significant odour expected during normal operation | Enclosed system with vapour return valve on bulk intake tanks |  | Can add to ongoing inspections if required |
| VOC’s |  |  | Cyclopentane | Any VOC’s in the formula are in minimal amounts and have less than trivial environmental impact. |

* cyclopentane is the only ingredient with a documented GWP (~10) and potential POCP (likely low). All ingredients, including cyclopentane, have an ODP of 0. The other ingredients (reaction mass, polypropylenglycol, formic acid, and amines) likely have negligible GWP and negligible to low POCP

Estimated total usage of Cyclopentane between 2024/2025   2000Kg

Annual monitoring of all LEV and extraction and air quality carried out by a leading environmental consultancy.

Emission control

Emissions are controlled at every stage in the process to ensure minimal exposure if any.

Process control measures

- Vapour return valve on bulk deliveries

- Closed system for chemical mixture

- Sealed foaming JIGS

- Fast Cure time before material is inert

Operational Controls

- PPM (planned preventative maintenance) scheduled by qualified engineers, any specialized work LEV inspections air quality tests carried out by external qualified specialists.

- Electronic reporting system for accidents, incident and near misses currently being used to report any mechanical issues, we are developing an more engineering based reporting system that integrates with the PPM.